Lignin-PUD Hybrids for Sustainable Floor Care

### 1. Solution Level

The Green-Armor project is designated at Technology Readiness Level (TRL) 5. While the primary feedstocks—Bio-Polyurethane Dispersions (Bio-PUD) and Technical Lignin—are commercially available at TRL 9, their specific application as a high-speed burnishable floor finish has successfully moved from laboratory validation (TRL 4) to system validation in a relevant environment (TRL 5).

### 2. Problem & Opportunity

Current floor care relies on persistent synthetic polymers (Acrylic/PE) that shed non-degradable microplastics. Commission Regulation (EU) 2023/2055 mandates a restriction on intentionally added microplastics by 2028. Diversey has a strategic opportunity to lead the market with a "Strong in Use, Weak in Waste" bio-hybrid that provides high durability without the environmental persistence paradox.

### 3. Solution Overview

The solution utilizes a Lignin-Reinforced Bio-PUD. The system replaces non-degradable carbon-chain polymers with hydrolytically susceptible urethane linkages. The formulation features a 60% renewable carbon matrix, a lignin-based aromatic structural "rebar," and a carnauba-cellulose nano-emulsion to ensure a COF≥0.60.

### 4. Solution Feasibility

Economic feasibility is achieved via "Upcycling Logic." By utilizing Technical Lignin—a high-volume industrial byproduct—the project achieves a concentrate cost of $9.22/kg, well below the stakeholder threshold of $25.00/kg.

### 5. Experience

The project team brings 15+ years of expertise in Colloidal Chemistry and Anionic Stabilization. We have successfully transitioned industrial coating lines from synthetic to bio-hybrid platforms in the wood and textile sectors using the same stabilization protocols proposed here.

### 6. Solution Risks

- Stability: Potential for pH-related precipitation. Mitigation: Implementation of a Gate 1 titration protocol and a 12-week accelerated aging schedule.

- Aesthetics: Lignin-induced ambering. Mitigation: Utilization of refined "Organosolv" lignin fractions for maximum film clarity.

### 7. Timeline, Capability, and Costs

- Timeline: 15 Months (Stabilization → ASTM Validation → Field Pilots).

- Capability: 100% compatible with existing Diversey high-shear mixing assets; no new capital expenditure required.

- Costs: Estimated R&D budget of $85,000 – $110,000.

### 8. Online References

- European Commission. (2023). Regulation (EU) 2023/2055. Access Link

- Fortune Business Insights. (2025). Lignin Market Analysis. Access Link

- ChemAnalyst. (2026). Bio-Polyol Price Index. Access Link