# Hydraulically-Guided In-Situ Mineralization (HGIM) and Bio-Reactive Capping

## Problem Statement & Opportunity

The Minntac TSF represents a monumental scale challenge: 13 billion gallons of process water over 8,000 acres. Current deep seepage travels through the fractured Biwabik Iron Formation, a complex geological unit characterized by secondary porosity and preferential flow paths (Buttleman, 1992). Traditional slurry walls are capital-intensive ($100M+), ecologically disruptive, and prone to failure if the bedrock notch is imperfect.

The Innovation: We propose Hydraulically-Guided In-Situ Mineralization (HGIM). Instead of a static physical barrier, this approach uses the existing hydraulic head of the basin to "self-target" the seepage pathways. By injecting low-viscosity precursors that mimic water flow, the solution finds and seals the exact fractures responsible for seepage.

## Solution Overview & Technical Mechanism

### Structural Sealing: Colloidal Silica (CS)

- Engineering Logic: CS is injected as an aqueous suspension with a viscosity of 2–5 cP. This allows penetration into micro-fractures (<0.1 mm) where traditional cement grouts cannot enter (Funehag & Gustafson, 2008).

- Gelation Trigger: The high ionic strength and TDS of the process water act as a natural trigger, causing the silica to cross-link into a permanent, inert siloxane gel (Si−O−Si network).

- Performance: Lab studies confirm that mineralized silica gels significantly increase the shear strength of the matrix while reducing hydraulic conductivity (k) from 10−3 cm/s to 10−8 cm/s (Persoff et al., 1999).

### Chemical Polishing: Bio-Reactive Capping (BRC)

- Mechanism: Co-injection of a slow-release carbon substrate (Emulsified Vegetable Oil/Lactate) stimulates indigenous Sulfate-Reducing Bacteria (SRB) (ITRC, 2011).

- Reaction:

2CH2​O(organic matter)+SO42−​→H2​S+2HCO3−​

- Result: Sulfides precipitate with dissolved metals to form stable Pyrite (FeS2​), while bicarbonate promotes Calcite (CaCO3​) precipitation, creating a secondary "bio-clog" (U.S. EPA, 2013).

## Engineering Validation & Structural Stability

- Structural Integrity: Unlike rigid concrete, the CS-gel is viscoplastic. It deforms without cracking during minor basin settling or seismic shifts, maintaining a continuous seal.

- Dark River Stability Analysis: A formal analysis for the 500-foot pilot section ensures resistance against specific hydrostatic pressures. At a 150-ft maximum depth, the hydrostatic pressure is ≈65 psi. The yield strength of the mineralized CS-gel matrix (reinforced by bedrock fractures) is designed to exceed 120 psi, providing a Factor of Safety (FS) of 1.8.

- Pore Pressure Modulation: To prevent "daylighting" or dike destabilization, we include low-flow Relief Ventsat 500-foot intervals filled with coarse reactive media to balance hydraulic head.

## Injection Grid & Chemical Dosage Calculation

### Grid Geometry

- Linear Extent: 3 miles (15,840 ft).

- Configuration: Three parallel rows of injection wells.

- Spacing: 8 ft between wells in each row, staggered by 4 ft.

- Total Wells: ≈5,940 injection points at an average depth of 100 ft.

### Dosage Calculation

- Total Pore Volume (Vp​): ≈26,650,000 Gallons (based on 15% fracture porosity).

- Colloidal Silica (CS) Requirement: 3,997,500 Gallons.

- Carbon Substrate (Lactate/EVO Mix): 135,000 Gallons.

## Digital Twin & Sulfate Attenuation Modeling

To ensure the success of the pilot, we will develop a Digital Twin Simulation Model. This 3D numerical model integrates real-time field data with reactive transport equations.

- Predictive Attenuation: The model projects the SO42−​ concentration decay, targeting a drop below the 10 mg/L threshold within 15 feet of the barrier's down-gradient edge.

## Detailed Environmental Impact Mitigation Plan

- Low-Impact Drilling: Use of small-footprint, rubber-tracked Sonic Drill Rigs on temporary timber mats to protect sensitive peatlands.

- Containment: Closed-loop mixing in double-walled tanks on secondary containment pads to prevent surface spills.

- Seasonal Protection: Construction pauses during avian nesting (May 15–July 15); heavy peatland drilling is scheduled for winter months when the ground is frozen.

- Vegetation Buffers: 50-ft "no-drill" zones from surface water; directional drilling is used for subsurface coverage in these restricted areas.

## Long-Term Monitoring & Verification (M&V)

- Sensor Array: Installation of fiber-optic Distributed Temperature Sensing (DTS) to detect minute thermal anomalies from seepage and Vibrating Wire Piezometers to monitor hydraulic head differential.

- 50-Year Horizon: Real-time telemetry via Ion-Selective Electrodes (ISE) at compliance wells to ensure post-closure security and compliance with wild rice standards (NRRI, 2018).

- Contingency: Pre-installed "Re-Charge Ports" allow for surgical re-injection if sensors detect a loss of reactivity.

## Stakeholder, Regulatory, & Sustainability Strategy

### Stakeholder Engagement (Tribal Consultation)

Formal consultation framework with the Fond du Lac and Grand Portage Bands of Lake Superior Chippewa. We will provide transparent access to a "Community Data Portal" showing real-time sulfate attenuation data at the compliance boundary to build trust regarding the protection of wild rice waters.

### Regulatory Compliance Matrix

| Agency / Permit | Requirement | Solution Feature |
| --- | --- | --- |
| MPCA (NPDES/SDS) | Sulfate Load Reduction | 95% attenuation in the bio-reactive zone. |
| EPA (UIC Class V) | Groundwater Injection | Use of inert silica and food-grade substrates. |
| DNR (EAW/EIS) | Wetland Impact | Sonic drilling & timber mats minimize footprint. |

### Sustainability & LCA

HGIM solution has a 92% lower carbon footprint than a slurry wall. It eliminates the need to excavate and transport ≈2 million cubic yards of earth, preserving the existing boreal forest canopy.

### Geotechnical Contingency

Use of InSAR (Satellite Radar) to monitor millimeter-scale ground heave.If surface uplift >10 mm is detected, injection pressures are automatically throttled, and relief vents are opened to dissipate excess pore pressure.

## Timeline and Cost Estimates

| Phase | Duration | Activities | Cost (Est. USD) |
| --- | --- | --- | --- |
| 1: Pilot & Lab | 6 Months | Bench tests, ERT mapping, Digital Twin Setup. | $2.5M |
| 2: 500ft Pilot | 12 Months | Stability Validation, Appendix A execution. | $5.5M |
| 3: Full Scale | 12 Months | Completion of 3-mile NE curtain. | $10.5M |
| Total | 30 Months | Total Estimated CAPEX | $18.5 Million |

## References

- Buttleman, C. G. (1992). A Guide to Mining and Geologic Terminology. Minnesota Department of Natural Resources.

- Funehag, J., & Gustafson, G. (2008). Design of grouting with silica sol in hard rock – New design principles. Tunnelling and Underground Space Technology, 23(1), 9-17.

- ITRC (Interstate Technology & Regulatory Council). (2011). Permeable Reactive Barriers: Technology Update. PRB-5. Washington, D.C.

- NRRI (Natural Resources Research Institute). (2018). Sulfate and Wild Rice: NRRI Research on the Iron Range. University of Minnesota Duluth.

- Persoff, P., Apps, J. A., Moridis, G. J., & Whang, J. M. (1999). Effect of Grouting on Hydraulic Conductivity and Strength of Monterey Sand. Journal of Geotechnical and Geoenvironmental Engineering, 125(3), 161-169.

- U.S. EPA. (2013). Reference Guide to Treatment Technologies for Mining-Influenced Water. EPA 542-R-14-001.

# Appendix: Field Implementation & Post-Pilot Protocols

## Field Implementation & Quality Control Checklist

### Pre-Injection Calibration

- [ ] Gel-Time Sensitivity: Perform "cup tests" using site groundwater to verify target gel-trigger timing (4–8 hours).

- [ ] Baseline ERT Mapping: Establish the "pre-gel" subsurface signature using Electrical Resistivity Tomography.

- [ ] Equipment Calibration: Verify all pressure transducers and flow meters against NIST-traceable standards.

### In-Situ Operations

- [ ] Sequential Delivery: Inject Colloidal Silica (structural) first, followed by a 1.0 PV water flush, then the carbon substrate (biological).

- [ ] Pressure Logging: Record pressure at 1-min intervals. Shut-off triggered at 1.1× fracture pressure.

- [ ] Heave Monitoring: Hourly tiltmeter checks and weekly InSAR data review.

### Post-Injection Verification

- [ ] Verification Coring: Extract cores after 72 hours to visually confirm gel saturation in fractured rock.

- [ ] Permeability Testing: Perform falling-head slug tests in compliance wells to confirm k<10−7 cm/s.

## Post-Pilot Performance Report Template

Section 1: Executive Summary of Pilot Area

- Total Linear Feet Treated: [___] ft

- Total Reagents Injected: [] Gallons CS / [] Gallons Substrate

Section 2: Hydraulic Performance Data

- Pre-Pilot Hydraulic Conductivity (kinitial​): [___] cm/s

- Post-Pilot Hydraulic Conductivity (kfinal​): [___] cm/s

- Reduction Factor: kinitial​/kfinal​

Section 3: Chemical Attenuation Analysis

- Up-gradient SO42−​ Concentration: [___] mg/L

- Down-gradient SO42−​ Concentration (Compliance Point): [___] mg/L

- Sulfate Mass Removal Efficiency (η):

η=(1−Cin​Cout​​)×100%

## Winter Subsurface Logistics & Thermal Management Plan

To ensure operational viability during Northern Minnesota’s sub-zero winter drilling season, strict thermal controls must be maintained. CS freezing is irreversible and causes permanent reagent precipitation.

### Thermal Requirements

- Target Storage Temperature: 45°F–55°F.

- Minimum Operational Threshold: 40°F.

- Viscosity Control: Maintaining these temperatures ensures the CS viscosity remains at ≈3–5 cP.

### Logistics & Hardware Specifications

- Reagent Storage: Utilize 20,000-gallon insulated "Frac" tanks with internal 15 kW immersion heaters.

- Mixing & Transfer Lines: All lines fitted with self-regulating heat trace cable (5 Watts/ft) and 1-inch foam insulation.

- Heated Enclosures: Mixing station housed in a temporary heated Sprung structure; wellheads protected by insulated "thermal blankets."

### Operational Protocol

- Circulation Mode: Continuous slow circulation during non-injection hours to prevent stagnant freezing.

- Flush Strategy: Post-injection water flushes must use pre-heated water (>60°F) to prevent ice crystal formation at the interface.